1. Introduction

The goal on this assignment is develop skills of creating basic and composite function blocks. The task is to implement functions from IEC 61499 and verify and validate implemented blocks.

2. Basic Function Blocks

- Describe the purpose of the Function Block.

- Show the event and data interface.

- Explain the Execution Control Chart (ECC).

- Briefly describe the algorithms used.

- Demonstrate the Function Block executing.

- Explain how the observed behaviour matches the supplied specification.

Implemented Function blocks: Increasing counter and RS flip-flop

Increasing counter:

Requirements:

The E_CTU block shall set Q to TRUE when predefined value of CV is reached. When Reset is triggered the E_CTU shall stop counting and trigger RO event. The Block must follow this timing diagram:

DIAGRAM

Implementation:

Interface consists of 2 input events CU and R and input INT value PV associated with CU event. Output consists of 2 events CUO and RO. 2 output data Q(BOOL) and CV(INT) both associated with CUO and RO events.

Execution control chart consists of 3 states (START,COUNT and RESET). Transitions are showed on screenshot

SCREENSHOT

State RESET triggers event RO and algorithm A_RESET is executed. Algorithm sets Q to false and CV to 0

State COUNT triggers event CUO and executes A_COUNT. Algorithm increase CV by 1 , Sets CU back to False  and then checks if CV is reached  PV. If so  is set on True, else Q sets on false.

Testing, verification and validation:

We can check block in Debug mode. Block works correctly and the way we defined in time diagram and copies the library block behavior

RS flip-flop:

Requirements:

Library implements E_RS and E_SR with same functionality. In assignment E_RS is implemented.

The E_RS block shall set Q to TRUE when event triggered on S input. Q should be reset to 0 when event R is triggered. Output event is triggered when Q value is changed. The Block must follow this timing diagram:

DIAGRAM

Implementation:

Interface consists of 2 input events S and R and no input. Output consists of one EO and BOOL value Q associated to it .

Execution control chart consists of 3 states (START,SET and RESET). Transitions are showed on screenshot

SCREENSHOT

State SET triggers event EO and algorithm A_SET is executed. Algorithm sets Q to 1.

State RESET triggers event EO and algorithm A_RESET is executed. Algorithm sets Q to 0.

Testing, verification and validation:

We can check block in Debug mode. Block works correctly and the way we defined in time diagram and copies the library block behavior

3. Composite Function Blocks (4–6 minutes)

- Describe the purpose of the Composite Function Block.

- Show the Function Block network.

- Explain how the Basic Function Blocks and Standard Library Function Blocks work together.

- Demonstrate the behaviour.

- Explain how the implementation satisfies the supplied specification.

Implemented Composete Function blocks: Event Train and Boolean Falling edge

E_TRAIN:

Requirements:

After event Start is triggered event EO shall be triggered N times every delayTime interval. After N time triggered EO or STOP event occurred, block shall stop.  The Block must follow this timing diagram:

DIAGRAM

Note: library implemented E_Train also requires STOP event after every START event

Implementation:

Interface consists of 2 input events START and STOP. Input UINT value N associated with START and STOP event. Input value delayTime(type TIME) is associated with START event. Output consists EO. Output data CV(UINT) associated with EO.

Function block network is showed on screenshot.

SCREENSHOT

Network consists of Basic function blocks:

E_cycle that triggers time intervals

E_CTU couns how much times E_CYCLE triggers output

E_PERMIT triggers stop and reset when N value is reached.

TWO_I_TWO_O is composite block consist of E_MERGE and E_SPLIT to make block with two inputs and output  events

SCREENSHOT

Testing, verification and validation:

We can check block in Application using Watch function and running simulation of network. Block works correctly and the way we defined in time diagram .

E_F_TRIGGER:

Requirements:

EO is triggered when transition QI from TRUE to FALSE is detected. If QI does not change, or changes from FALSE to TRUE, no EO is fired.  The Block must follow this timing diagram:

DIAGRAM

Implementation:

Interface consists of EI and QI (BOOL) is assiciated with this event. Output consists EO and no output variable.

Function block network is showed on screenshot.

SCREENSHOT

Network consists of Basic function blocks:

E_D_FF reacts on changing of QI

E_PERMIT with NOT triggers EO when D flip-flop detected on LOW state

Testing, verification and validation:

We can check block in Application using Watch function and running simulation of network. Block works correctly and the way we defined in time diagram and copies behavior of library block .